| Handy Sheet 2.0 27/April/98 file handy2.zip One-page pilot guide to be carried in all your virtual flights SEE "README" FILE | By Wagner B. Beskow W.B.Beskow@massey.ac.nz |
| --- | --- |

| Standard 3o Glide Slope |  |
| --- | --- |
| DIST (NM) | ALT (FT) |
| 15 | 4776 |
| 14 | 4458 |
| 13 | 4139 |
| 12 | 3821 |
| 11 | 3502 |
| 10 | 3184 |
| 9 | 2866 |
| 8 | 2547 |
| 7 | 2229 |
| 6 | 1910 |
| 5 | 1592 |
| 4 | 1274 |
| 3 | 955 |
| 2 | 637 |
| 1 | 318 |
| 0.5 | 159 |
| 0 | TD |

Distance from touchdown

Altitude AGL

Cruising Altitudes

Eastbound Mag Course

IFR  odd thousands

VFR  odd + 500FT

Westbound Mag Course

IFR  even thousands

VFR  even + 500FT

ATC: When available, contact controllers in this order: FSS, GRD, TWR, DEP, CTR, APP, TWR, GRD. Keep talk to the minimum necessary. Always read back your instructions, it's for YOUR safety.

Fly your a/c first, ctc/reply second. Do not execute an ATC instruction without thinking about it. Suggest an alternative if the last instruction can put your crew and your passengers at risk. Ultimately, YOU, as the pilot in command, are the one responsible for your a/c.

UNICOM: When ATC not available you should comm with other pilots. Listen, scan the skies and tell others what you will do next [MP chat window].

Charts: Always have them, at least the ones produced with Flight Planners.

What is the Alt of your airport of destination?

Obstacles?  !!!!

Crosswind Lndg: let nose point to the wind (crab angle); just before flare align a/c with rwy using rudder and apply opposite aileron concomitantly to counteract (slip).

Don't debug anything on final

GO AROUND !

A small correction early is better than a big correction late

Absolutely Indispensable Check List Points

Handy Conversions  and Equivalents

1NM = 1.1516SM = 1.853km

1SM = 0.8683NM = 1.609km

1FT = 0.305m

1KTS = 1NM/hour

inch Hg 			         mm Hg

TAS = (Alt/100,000 x 2 + 1) x IAS      (approx.)

[From TAS = IAS + 2% per 1000ft Alt]

FUEL       (follow multiply, back divide)

Litres			             pounds

US gallons		           kilograms

Imperial gallons

Altimeter set to 29.92inHg (1013mmHg) above FL180

Set to local pressure  (listen ATIS) when below FL180

Pre-clearance	Flight Plan

Fuel

Pre-taxi		Nav Radios Set

Wind Dir + Speed (ATIS)

x 33.86

÷ 33.86

Engine(s)   OK

A/P OFF

A/P VS Set to Zero or +

Pre-T/O		Flaps T/O Pos.

Elev. Trim T/O pos.

1.58

Rudder Centred

2.2

0.72

Rolling		Engine(s)   OK

6.0

3.8

Vr

2.72

Climb		Gear Up

7.2

4.5

Flaps (up as rqd.)

3.27

1.2

Cruise		Flaps Up

Cruising Speed

Descent		Reduce Speed

oC           oF

-50     -58

-30     -22

-25     -13

-20       -4

-15        5

-10      14

- 5       23

0       32

5       41

10       50

15       59

20       68

25       77

30       86

35       95

40     104

oF=oC x1.8+32

oC=oF-32/1.8

A   ALPHA		. —

B   BRAVO		— . . .

C   CHARLIE	— . —  .

D   DELTA		— . .

E   ECHO		.

F   FOXTROT	. . —  .

G   GOLF		—  — .

H   HOTEL		. . . .

I    INDIA		. .

J    JULIET		. — — —

K   KILO		—  . —

L    LIMA		. —  .

M   MIKE		—  —

N    NOVEMBER	— .

O    OSCAR		—  —  —

P     PAPA		. —  —  .

Q    QUEBEC	— —  . —

R    ROMEO	. — .

S    SIERRA		. . .

T    TANGO		—

U   UNIFORM	. . —

V   VICTOR	. . . —

W  WISKEY	. — —

X   XRAY		— . . —

Y   YANKEE	— . — —

Z   ZULU		— — . .

1   . — — — —

2   . . — — —

3   . . . — —

4   . . . . —

5   . . . . .

6   — . . . . .

7   — — . . .

8   — — — . .

9   — — — — .

0   — — — — —

Flaps as Rqd.

Heat ON if OAT<10oC/50oF

Approach	SPEED and ALT

Decide G/S (ILS x Visual)

ILS Freq. Set + Confirmed

OBS Set to Rwy Course

Final App.	Check G/S

App. Speed

Gear Down (OM or earlier)

Flaps Down to Lndg Pos.

Landing		Target Speed

Flare*  at 15-40FT AGL

Nose Gear on ground, Break

Reverse Down to 60KIAS

(when applicable)

* power to idle, raise nose to 6o (approx.)

Reciprocal

Rwys Help

36    18

01    19

02    20

03    21

04    22

05    23

06    24

07    25

08    26

09    27

10    28

11    29

12    30

13    31

14    32

15    33

16    34

17    35

18    36

KNOW YOUR AIRCRAFT

Indispensable: V speeds, flaps x speeds, stall speeds, fuel consumption, app speed, target touchdown speed.

N(+)

W(-) + E(+)

S(-)

Main V speeds

V1 T/O decision speed

Va  design maneuvering speed

Vfe  max flap extension speed (top of white arc on ASI)

Vmca  min control speed

Vne  never-exceed speed

Vmo  max operating speed

Vno  normal operating speed (top of green arc on ASI)

Vr  rotate

Vso  stalling speed at MTWA, ie landing conf., sea lvl,                                                                       ISA conditions (top of white arc on ASI)

Vx  best angle of climb speed on all engines

Vxse  best engine-out angle of climb speed

Vy  best rate of climb speed on all engines

Vyse  best engine-out rate of climb speed

Speed expressed in IAS (Indicated Air Speed)

UTC=Z=GMT

It is better to be on the ground wishing you were flying, rather than up in the air wishing you were on the ground

EMERGENCY        checklist

Aviate best-glide___KIAS and trim; configure

Navigate pick a field, ahead if possible. Emgncy glide path is steeper !

Investigate heat, pump, tanks, temperature, pressure

Communicate ATC + 7700

Secure gear up for short, soft or water (if down, only close to TD), throttle, power, master, tanks - OFF; belt up !

Collision injuries all depend on the square of airspeed

High+Fast safer  than Low+Slow